XUM ENTERPRISE

User Manual

www.drmem.com

support@drmem.com

+44 161 877 0030

PAGE 1

1.

BEFORE YOU START

    - Location For Your New PC

CONTENTS

6.

MOTHERBOARD & REMOTE

6.1 	Motherboard Specifications

6.2 	Remote Control

PAGE 10

PAGE 9

5.

HEADPHONES & SPEAKERS

5.1 	Headphones & Speakers

NETWORK, MOUSE & KEYBOARD

4.1 	Network

4.2 	Mouse & Keyboard

4.3 	Useful Key Bindings

4.

PAGE 6

PAGE 12

8.

PAGE 11

7.

HOW TO RETURN A PC

8.1 	Contact us

8.2 	Returns Checklist

8.3 	Warranty

BIOS BEEP GUIDE

7.1 	BIOS Beep Guide

PAGE 3

3.

CONNECTING A MONITOR

3.1 	Single Monitor

3.2 	Dual Monitors

POWERING YOUR COMPUTER

2.1 	Powering Your Computer

PAGE 2

2.

BEFORE YOU START

  - Location For Your New PC

- Decide where you are going to locate the PC, as gaming PCs require adequate ventilation.

- Place the PC in an area with plenty of unobstructed space so the fans can circulate air freely.

- Be careful to leave at least 30 cm at the rear & top of the PC so heat can be properly dissipated.

- Be careful not to block, or cover, any of the fans at the front of the PC as these vents provide cool air flow.

- Make sure not to sit the PC near radiators, hot pipes, or any other heat sources.

PAGE 1

POWERING YOUR COMPUTER

  - Powering Your Computer

PAGE 2

ON | OFF

- Plug the power cable in at the rear of the PC (Make sure the I/O switch is OFF).

- Turn on power to the PC using the switch on the rear.

- Press the power button on the top of the PC.

- Dedicated Graphics Card (DGC) Ports (Use these if present).

DVI – D | I

CONNECTING YOUR MONITOR

- This is the side of the Graphics Card.

- This is the side of the Motherboard; Integrated Graphics Ports (IGP) are found here.

- Integrated Graphics Ports WILL NOT give a display if there is a DGC Installed (Below).

- Gaming PCs with Dedicated Graphics Cards (DGC).

PAGE 3

Display Cables (KEY)

HDMI

VGA

Display Port

  - Single Monitor

CONNECTING YOUR MONITOR

  - Dual Monitors

- You may want to maximize the screen size by having twin monitors.

- This is very easy to do with XUM Gaming PCs. You will need the following cables to connect the GPU to the monitors.

- See the table below to check what cables you need depending on the graphics card.

- You should connect both the primary display and secondary display into the graphics card if you are gaming on both monitors, if both monitors are running different programs, you should connect the secondary display into the IGP on the motherboard.

- You may need a second adapter or cable if you wish to convert a connection, DVI to HDMI for example.

PAGE 4

- When you have both monitors connected, the Windows desktop will be displayed on both screens.

- If you are not getting a signal on the 2nd monitor, check the connection and power to the 2nd monitor and check the OSD (on screen display) on the monitor or TV.

- Press the Windows Key and P to bring up the configuration choice.

- The most used option is Extend, which extends your display across 2 screens.

- The next most used option is Duplicate which duplicates the display on both screens.

- Alternatively, click on the Settings cog, then System, then Display.

- With Extend you can choose screen 1 as the gaming screen and screen 2 for internet browsing, or you can extend the gaming display across both screens for a widescreen experience.

System

P

CONNECTING YOUR MONITOR

PAGE 5

  - Dual Monitors

Ethernet Port

- Attach the Wi-Fi Antennas to the gold sockets at the rear of the PC as shown below.

- Or use the Wired Ethernet port (RJ-45) on the motherboard.

- For online gaming, wired is always best.

- Connect to a home network via Wi-Fi or Ethernet and you should be ready to start downloading and playing games!

NETWORK

PAGE 6

4.1 Network

MOUSE AND KEYBOARD

- Plug in the mouse and keyboard into the black USB 2 ports.

- Reserve the blue USB 3 ports for high-speed devices, such as external hard drives or webcams.

- You can also control the case lighting with the included remote control, or (on some cases) the LED button on top of the case.

4.2 Mouse & Keyboard

PAGE 7

USB 2 Ports

USB 3 Ports

PAGE 8

4.3 Useful Key Bindings

Website

FN

F9

Adjustable DPI / Mouse Sensitivity

SL

LED’s ON/OFF

F10

F11

F12

Mail

Folder

Help

FN

FN

FN

Play / Pause

Next Track

Prev Track

Vol Down

Vol Up

Stop

Mute

Player

FN

F3

FN

F2

FN

F1

FN

F4

FN

F5

FN

F6

F7

FN

FN

F8

Useful Key Bindings

MOUSE AND KEYBOARD

HEADPHONES & SPEAKERS

- Pink is for Microphone | Blue is Line in | Green is Speakers / Headphones.

- If you are using a gaming headset with a single audio jack you will need an audio splitter.

- If you choose to use Bluetooth, you will need a USB Bluetooth adapter as this machine does not have built-in Bluetooth.

- Gaming headsets with a single audio jack require an audio splitter. Two rings are for stereo. Three rings are for stereo and a microphone.

- USB headphones often have virtual 3D sound capabilities and can be better for gaming.

5.1 Headphones & Speakers

PAGE 9

6.2 Remote Control

6.1 Motherboard Specifications

Remote Control

MOTHERBOARD SPECIFICATIONS

PAGE 10

BIOS Beeper Guide

PAGE 11

- 1 Long Beep

  - No Issues

- 1 Long, 2 Short Beeps

  - Failure in video system – E.g., new hardware, a malfunctioning Fan, a CD / Disk, or power supply.

- 1 Long, 3 Short Beeps

  - RAM / Memory failure

- 1 Long, 8 Short Beeps

  - Video adapter missing / defective.

- 1 Short Beep

  - DRAM refresh failure

- 10 Short Beeps

  - CMOS shutdown Read/Write error

- 11 Short Beeps

  - Cache Memory error

7.1 BIOS Beeper Guide

PAGE 11

- 2 Short Beeps

  - Parity circuit failure

- 3 Short Beeps

  - Base 64K RAM failure

- 4 Short Beeps

  - System timer failure

- 5 Short Beeps

  - Process failure - The system CPU has failed

- 6 Short Beeps

  - Keyboard not present

- 7 Short Beeps

  - Fault in the CPU or motherboard

- 8 Short Beeps

  - Read / Write Test Failure E.g., new hardware, a malfunctioning Fan, a CD / Disk, or power supply.

- 9 Short Beeps

  - ROM BIOS checksum failure

This small device attaches to the motherboard and makes various beeping sounds based on any issues with the computer. Here is a breakdown of every different beep composition and what it means.

How to Return a PC

Returning For a Refund?

You can return a PC for a full refund if we have been notified within 30 days of the goods being received that you want to cancel the order. You have another 14  days to return the goods once we have been notified (See Warranty first).

Returning For a Repair/Replacement?

You can return a PC for a repair/replacement if we have been notified within 365 days of the goods being  received (See Warranty first).

£

+44 161 877 0030

support@drmem.com

PAGE 12

See full warranty policy on Page 14

8.1 Contact us

1

2

1

Return Checklist

Back-up your HDD and/or SSD data before returning. We are not responsible for  any loss of data that may occur during repair or replacement.

Only return the PC itself. Do not include any cables or accessories.

Remove the user password from your PC or contact us with user details before returning.

Packaging the PC

You will need to pack your XUM PC safely and securely before shipping it back to us. We are not responsible for anything damaged in transit. It is best to return the item as originally packaged. If this is not possible follow the instructions (right):

Double Box the PC

Protect your PC so not to get damaged in transit by packing it securely.

- Place the PC in a cardboard box (Box 1), filling any empty space with packing material such as bubble  wrap to make sure it doesn't move around, then seal Box 1 with packing tape.

- Place Box 1 into a second outer box (Box 2), also filling any empty space with packing material such as bubble wrap to make sure it doesn't move around, then seal the box with packing tape.

- Contact Us to arrange a collection time and date that suits you.

A courier will collect and label the  parcel between 9am – 5pm, Weekdays (excluding bank holidays).

8.2 Returns Checklist

PAGE 13

How to Return a PC

8.3 Warranty

Warranty

What is not covered under Warranty?

- This Guarantee does not cover

your data, or any peripherals that are not manufactured by XUM and were not originally included.

- Any damage sustained to the PC

in transit back to us (see page 13).

When is Warranty Void?

- The warranty period has expired

- Failure or damage due to misuse such as, water damage, physical damage.

- Altering the product unless expressly permitted in writing by us.

- The following components have been replaced by a third party: HDD, SSD, RAM, CPU, GPU, MOTHERBOARD, CASE, PSU

- The product has been taken to a third-party computer repair shop.

- Motherboard, PSU, or CPU failure due to overclocking.

XUM guarantees that this product is free from defects in materials and workmanship that result in product failure during normal usage in accordance with the terms set out on this page and   will, for a period of 1 year from the date of original purchase.

We guarantee repair or replacement of any component  that is part of this product, free of charge, where it is faulty due to defective materials or workmanship. Replacements will be with new or, refurbished components.

If a repair/replacement is not possible we reserve the right to offer an alternative of equal or greater value (at the customers’ expense) or a partial refund proportional to the cost of the part.

Warranty is non transferrable and applies to the original purchaser only. Proof of purchase is required with all potential warranty claims.

PAGE 14

Support Page

www.support.drmem.com

- You should now be good to go!

- Windows is pre-activated for you.

- If you run into an issue, then please see our online guides for troubleshooting tips (Scan QR Code Above).

- You can also get in contact with us via email or phone from Monday – Friday (Excluding Bank Holidays).

PAGE 15

support@drmem.com

www.drmem.com

+44 161 877 0030

PAGE 12

www.support.drmem.com

Follow us on Social Media

PAGE 16

@xum-enterprise-ltd

@Dr_Memory_UK

Twitter

@xum_enterprise

Instagram

LinkedIn

@DrMemoryUK

Facebook

PAGE 12

PAGE 12